CONSTRUCTION MANAGEMENT III PROJECT PHASE I INFORMATION PHASE

In the first phase of a value engineering study, project information are collected and summarized in a way that can be easily utilized by the multidisciplinary team performing the analysis. Information is needed regarding the various functions of the project. This assignment should help each team in determining the type of information that is needed and the format in which the information should be presented. Each team should report on all the issues listed in this assignment.

FORMAT OF PHASE I REPORT

1. TITLE PAGE:

Project Name Location Team Members

2. PROJECT SCOPE

•  Project cost plan: Budget allocated by owner to the project

•  Design basis: Owner's program needs for net square feet of space and essential owner requirements

•  Schedule: Estimated construction period

3. PROJECT GOALS & PERFORMANCE LEVELS (Refer to fig. 3.16 and 3.18 of Textbook)

Project goals define the values of the owner and tell the value engineer what is to be achieved in the project. This list is important because it will be used to temper the outcome of economic analysis. For example, the list of goals that need to be considered may include:

• Image/ Performance

•  Security

•  Expansion

•  Energy conservation

•  Cost effectiveness

4. FAST DIAGRAM

The FAST diagram displays an analysis of the project basic function into several sub-functions. It also displays the (Why/How) relationships between these sub-functions. The FAST diagram development process forces the VE study team to focus on the identification of the project basic function and therefore usually produces a better functional analysis of the project. The development process opens up communications among the owner, designer, and. the VE study group.

The development of the FAST diagram is mainly recommended for complex projects whose functions are not well understood by the VE team even after drawings review and preliminary functional analysis. Each team will be required to submit a FAST diagram for its project The objective is to make sure that each team fully understands the basic function of its project and they are capable of developing a FAST diagram in case there is a need for one. It should be noted that the development of FAST diagram is a group effort and should not be done on an individual basis.

5. FUNCTIONAL ANALYSIS

In the functional analysis, the project components are defined as having basic or secondary functions. The objective is to give a feel for the percentage of the cost (or total area) that is being used for performing the basic functions as compared to that used for secondary functions. Once these costs (or areas) are identified, it becomes easier to reduce the cost (or area) of nonessential secondary functions while still providing the appeal necessary to permit the design to be attractive. The functional needs of the owner must be reviewed to ascertain that all major space elements of the facility are included.

6. FACILITY CONFIGURATION

In this section, a description of your building configuration (i.e. number of floors, height, perimeter, and volume) should be given. The objectives in determining configuration are for determining design parameters and for computing cost for various budget elements. The following should be calculated (if applicable):

- Site area

- Building Orientation

- Location of Building within the Site

- # of floors below grade

- # of floors above grade

- Basements gross area

- Ground Floor gross area

- Upper Floors gross area

- Total Gross building area

- Roof area

- Footprint area at grade

- Basement wall area

- Exterior wall area

- Basement volume

- Floor to floor height (Basement, ground floor, upper floor, special floors)

- Building shape: Draw a sketch of your building and examine the shape of your building with regard to energy conservation and freedom of design expression

- Typical floor perimeter, roof perimeter and basement perimeter

Examples of data obtained from building configuration

Foundation perimeter (LF) for grade beams or drain system = building perimeter

Slab on grade area = ground floor gross area

Excavation volume = volume of basement

Waterproofing area = basement wall area +slab on grade area

Length of parapet walls and flashing = roof perimeter

Site improvement area = site area - ground floor area

7. REVIEW OF FACILITY CONFIGURATION, FUNCTIONAL ANALYSIS & PROJECT SCOPE

In this section you should use the information you collected in the previous sections to recommend and discuss any changes in building configuration and project scope that you think will improve the project value. Examples of the things you need to do to complete this section include:

- Review area allocation and identify problem areas (i.e. areas that are too small or too large for the function intended (Figure 3.11 page 47). Recommend changes to the size of problem areas.

- Review grouping of functional areas and flow of people and materials and if applicable, recommend changes to the distribution of areas to allow for better circulation within the building. o Review site layout to identify any problems with accessibility, shape of building o Review building configuration to identify any constructability issues such as excessive number of angles and curvatures in exterior and interior walls.

- Include a discussion of potential problems of the selected systems and methods of preventing these problems from occurring.

- Develop a Cost Model for the original design of the project The cost model is presented in one page to summarize project cost information. It gives the VE study team a feel for the cost structure of the original design, and help the team in thinking of cheaper alternatives and in selecting areas having greatest saving potential. For a VE cost model, 2 types of costs are required: target cost (worth cost) and design costs. The target cost is the minimum cost believed possible for each block of the cost model and is based on team experience with similar buildings and historical cost files.

- Insert cost and worth values in the model for only the following systems:

•   Structural System

• Exterior wall system

• Roofing system